%N mfplan

%D Determine good frequency setups for multi-frequency synthesis.

%P rjs

%: utility

%B

MFPLAN helps determine observing frequencies for multi-frequency

synthesis experiments with the ATCA.

%A freq

Frequency bands (in GHz) to search for good mfs setups. This consists

of a number of pairs of frequencies, giving the lower and upper

limits on bands appropriate for observing. Several bands will often

be given to avoid frequencies which suffer from interference

or birdies. The nominal frequency bands of the AT receivers are:

1.35,1.78    (L band)

2.20,2.50    (S band)

4.544,6.336  (C band)

8.00,9.20    (X band)

The default is 4.544,6.336.

%A nfreq

Number of frequencies per configuration. Generally 2 or 4 are

the most usual numbers for the ATCA. Default is 2.

%A nants

Number of antennae to use. Sensible values are either 5 or 6.

Default is 6.

%A config

Array configuration. Several values can be given. Valid values are:

3.0a (6.0a), 3.0b (6.0b), 3.0c (6.0c), 3.0d (6.0d), 1.5a, 1.5b,

1.5c, 1.5d, 0.75a, 0.75b, 0.75c, 0.75d, 0.375, 0.122. These correspond

to the 14 standard ATCA configurations.

%A fixed

List of fixed frequencies that are fixed (not varied) by MFPLAN.

Usually these correspond to frequencies used in completed

observations. The first "nfreq" frequencies will be

associated with the first configuration, the second "nfreq"

frequencies correspond to the second configuration, etc.

The default is that all frequencies are varied.

%A options

Extra processing options. Several can be given, separated by commas.

Minimum match is used. Possible values are:

no128    Frequencies that are multiples of 128 MHz are to be

avoided. All resulting frequencies are odd multiples

of 64 MHz.

infinity Minimise the worst gap in the uv coverage. The default

is to minimise the rms gap in the uv coverage. The

infinity option is the best option if you have very good

uv coverage. Otherwise it produces poorer results.

colours  Plot each configuration with a different colour

%A device

PGPLOT device. If this is given, then a plot of the u-v coverage

is made. Default is no plot.