%N sinpoly

%D Fit polynomial baselines to autocorrelation data.

%P mchw

%: uv analysis

%B

SINPOLY - Fit polynomial baselines to autocorrelation data.

Polynomial fits for each antenna and polarization are subtracted

from the output averaged data.

%A vis

The name of the input uv-data file.

%A select

The normal uv selection commands. The default is use everything.

See the help on "select" for more details.

%A line

The normal uv linetype in the form:

line,nchan,start,width,step

See the help on "line" for more details. The default is all channels.

The output will consist of only spectral data.

%A interval

Time averaging interval, in minutes. The default is 0. (i.e. no

averaging).

%A out

The name of the output uv data set. No default.

%A options

This gives extra processing options. Several options can be given,

each separated by commas. They may be abbreviated to the minimum

needed to avoid ambiguity. Possible options are:

'nocal'      Do not apply the gains file. By default, SINPOLY

applies the gains file in copying the data.

'nopol'      Do not apply polarization corrections. By default

SINPOLY corrects for polarization cross-talk.

'window'     Fit polynomial to each spectral window.

Default is to fit all channels.

'dsb'        Fit polynomial to each sideband for double sideband

spectra. 'dsb' and 'window' are mutually exclusive.

%A device

PGPLOT device for polynomial fits. The default is to ask user.

%A nxy

Two values. Number of plot windows in x and y, Default nx=1, ny=nx.

%A npoly

The order of the Lagrange polynomial fit to the averaged uv-data.

Different values can be given for each window.

The maximum is 8th order polynomial. Default 0,0,0,0,0,0,0,0,0,0,0,0

%A badchan

Number of ranges of bad channels followed by list of up to 20 pairs

of numbers to specify range of channels to exclude in polyfit.

e.g. to exclude 2 center channels in eight 64-channel windows, use:

badchan=8,32,33,96,97,160,161,224,225,288,289,352,353,416,417,480,481

%A endchan

Number of channels to drop from window edges in polyfit. Default=4.