%N offpol

%D Generate ATCA primary beam polarimetric response.

%P rjs

%: utility

%B

OFFPOL generates images of the primary beam response (both total

intensity and polarimetric) of the ATCA antennas.  It performs a

simple simulation of an observation.

%A out

Output name template.  No default.

%A imsize

Two values, the image size.  Second value defaults to first.

Default 255.

%A cell

Two values, the image cell size in RA and Dec, in arcsec.

Second value defaults to first.  Each defaults separately to

twice the primary beam FWHM divided by imsize.

%A ra

Right ascension of the field centre.  (Used solely for setting

the output header, it has no effect on the computation.)

Default 0.

%A dec

Declination of the field centre.  Default -45 deg.

%A harange

Hour angle range to simulate - the start and end hour angles,

and the simulation step size.  The default is to simulate a

snapshot at 0 hours.  The default step size is 0.1 hours

(6 min) which might be inadequate for sources with a declination

near -30 deg.

%A freq

Frequency of interest (GHz).  The default is 1.384 GHz.

%A options

Task enrichment parameters. Several can be given.

raw      Generate images of the XX,YY,XY and YX responses,

rather than the Stokes responses.

subtract Subtract off the circularly symmetric portion of the

primary beam response from I, XX and YY responses.